Asset Tag Attach + Detach

Asset Tag Attach + Detach

How to install an asset tag onto equipment:

|  | Check the Cognosos asset tag placement guide to see the correct location and orientation of the tag per equipment type. |  |
| --- | --- | --- |
|  | Clean the area on the equipment where the tag will be installed with isopropyl alcohol wipe, or similar. Dry the cleaned area with a paper towel, or air dry. |  |
|  | Never submerse asset tag or equipment into liquids. |  |
|  | Peel the red adhesive backing from a corner and remove completely. |  |
|  | Press and hold the tag on the equipment for a minimum of 10 seconds. Make sure that the tag is securely placed on the selected equipment. |  |
|  | Associate tag in the Cognosos Asset Management Portal (https://portal.cognosos.net/). (See “attach tag” instructions the RTLS Healthcare User Guide or Healthcare Knowledge Base.) |  |

Need Assistance? Check out the Healthcare section of the Knowledge Base or contact us at support@cognosos.com.

How to detach an asset tag from equipment:

|  | Insert Cognosos key between the asset tag and the cradle mount. Clean both the asset tag and the cradle mount with isopropyl alcohol. |  |
| --- | --- | --- |
|  | Starting at the corner of the cradle mount, slide a metal edge scraper or flat head tool and gently apply even pressure away from yourself. Remove tag from equipment and store in a designated bin or drawer. |  |
|  | Clean the residual adhesive with isopropyl alcohol or light abrasive pad until no adhesive markings remain on the equipment. |  |
|  | Detach tag from the asset using the Cognosos Portal (https://portal.cognosos.net/). This is very important to keep the inventory and the equipment’s historical data accurate. (See “detach tag” instructions in the RTLS Healthcare User Guide or Knowledge Base.) If you need to continue monitoring the asset from which the tag was removed, secure a new tag onto the asset and attach the tag using the portal. |  |

Need Assistance? Check out the Healthcare section of the Knowledge Base or contact us at support@cognosos.com.